Our objective of this project is to continuously and remotely measure 3 rain gardens for the following parameters

- Cumulative Rainfall

- Depth of water in rain garden

  - Combine with drainage area to calculate timestep inflow volume

- Ambient temperature

- Relative Humidity

- Depth of flow in underdrain.

  - Use Manning’s equation to determine flow rate

We hoped to upload this data to a website using an HTTP Post through a cellular module from a solar powered station. Cellular method was abandoned because of inability to connect to the website. Pivoted to collecting data on SD card but data was quickly corrupted. We believe power supply was also an issue; the current solar panel can’t charge the station completely and the power dwindled.

For each project location, we have:

- 1 Waterproof Housing box

- 1 ISCO 674 Rain Gauge (0.01” tip)

- 1 Arduino UNO

- SIM7000 LTE CAT-M1 NB-IoT Cellular + GPS + Antenna Shield Kit for Arduino (SIM7000A)

- 1 Hologram.io SIM card

- MicroSD Card Adapter

- 2 GB microSD Card

- 2 pressure transducers rated for 5V and 30 psi

- 1 SHT31 outdoor Temp and Humidity Sensor (3.3-5V)

- 1 Grove Water Level sensor (10 cm)

- 1 6V 5W Solar panel

- 1 Schottky diode

- 1 Boost Converter

- 1 Buck Converter

- TP4056 Battery charging module

- 1 3.7V 6600mAh Li-ion battery

- Tons of jumper wires

- Breadbox

- Security Wire to connect sensors to station

If our outside help can figure out a way to continuously effectively power the system with all of the sensors + the SIM7000 module, while being able to attach packets of data over cellular, we should be in good shape.

- Do we need a different battery?

- Do we need a bigger solar panel?

- Do we need stronger wires?

- Do we need appropriate current delivered to each component of the system?

- Do we need proper coding to get the SIM7000 to deliver data?